# GlacialLight Expands Waterproof Lighting Series with 8” Down Light – GL-DL08-IP

## Versatile Ceiling Lighting Suited for Wet Indoor and Sheltered Outdoor Installations

March 18, 2014, Taipei, Taiwan – GlacialLight, a division of lighting and technology manufacturer GlacialTech, announces today the newest addition to its waterproof LED down light series. The GL-DL08-IP is an 8” waterproof down light for wet spaces, rated for dust and waterproof protection to IP54. Available in a variety of color temperatures to suit different environments, the DL08-IP can also come with 3-in-1 dimming (DL08DA-IP) and has DC input (DL08LD-IP) available.

### Ambience

As part of GlacialLight’s Cappella series of down lights, the GL-DL08-IP features a sleek, circular and lightweight design that fits in with contemporary surroundings. Color temperature options include from 3000K (Warm White), 4000K (Neutral White) and 5000K (Cool White). Light gently fades at the edges of the down light for a special atmospheric effect perfect for spaces such as saunas, spas, bathrooms or swimming pools.

### Versatile

The DL08-IP series of down lights has the versatility to suit a variety of indoor and outdoor applications. The down light comes in either AC or DC input (11-15V/22-30V) configurations. 3-in-1 dimming via 1-10V, PWM, or Resistor is available, as well as a non-dimming version. A wide 110 degree beam angle casts light evenly across any room or sheltered outdoor space.

### Specifications

DL08-IP (non-dimmable)

| Type | GL-DL08-IP-WW | GL-DL08-IP-NW | GL-DL08-IP-CW |
| --- | --- | --- | --- |
| Energy Used | 18W |  |  |
| Rated Life | 30,000 Hours |  |  |
| Input Voltage | 100-240V AC |  |  |
| Power Factor (PF) | 0.88@120V AC, 0.8@240V AC |  |  |
| CCT (color) | 3000K | 4000K | 5700K |
| Luminous Flux | 1000 lm | 1100 lm | 1200 lm |
| System Efficacy | 56 lm/W | 61 lm/W | 67 lm/W |
| CRI (Minimum) | 80 | 80 | 80 |
| Beam Angle | 110° |  |  |
| Dimension | Ф 208 x 25 mm |  |  |
| IP Rating | IP54 |  |  |
| Operation Temperature | -20°C ~ +40°C |  |  |
| Bundled Driver | GP-HS15P-22C1 |  |  |
| Net Weight | 660 g (with driver) |  |  |
| Package | 10 PCS / Carton |  |  |

DL08DA-IP (DC dimmable: 1-10V/PWM/Resistor)

| Type | GL-DL08DA-IP-WW | GL-DL08DA-IP-NW | GL-DL08DA-IP-CW |
| --- | --- | --- | --- |
| Energy Used | 18W |  |  |
| Rated Life | 30,000 Hours |  |  |
| Input Voltage | 100-240V AC |  |  |
| Power Factor (PF) | 0.88@120V AC, 0.8@240V AC without dimmer |  |  |
| CCT (color) | 3000K | 4000K | 5700K |
| Luminous Flux | 1000 lm | 1100 lm | 1200 lm |
| System Efficacy | 56 lm/W | 61 lm/W | 67 lm/W |
| CRI (Minimum) | 80 | 80 | 80 |
| Beam Angle | 110° |  |  |
| Dimension | Ф 208 x 25 mm |  |  |
| IP Rating | IP54 |  |  |
| Operation Temperature | -20°C ~ +40°C |  |  |
| Bundled Driver | GP-HS15P-24CA (for 1-10V/PWM/Resistor) |  |  |
| Net Weight | 660 g (with driver) |  |  |
| Package | 10 PCS / Carton |  |  |

DL08LD-IP (DC input)

| Type | GL-DL08LD-IP-WW | GL-DL08LD-IP-NW | GL-DL08LD-IP-CW |
| --- | --- | --- | --- |
| Energy Used | 18W |  |  |
| Rated Life | 30,000 Hours |  |  |
| Input Voltage | 11-15V DC / 22-30V DC |  |  |
| CCT (color) | 3000K | 4000K | 5700K |
| Luminous Flux | 1000 lm | 1100 lm | 1200 lm |
| System Efficacy | 56 lm/W | 61 lm/W | 67 lm/W |
| CRI (Minimum) | 80 | 80 | 80 |
| Beam Angle | 110° |  |  |
| Dimension | Ф 208 x 25 mm |  |  |
| IP Rating | IP54 |  |  |
| Operation Temperature | -20°C ~ +40°C |  |  |
| Bundled Driver | GP-LD15-24C (Driver Output: Constant Current 700mA, 25V) |  |  |
| Net Weight | 600 g (with driver) |  |  |
| Package | 10 PCS / Carton |  |  |

### Features:

- For indoor or sheltered outdoor ceiling lighting applications

- Waterproof (IP54)

- All-in-one fixture

- Light flow smoothly fades around the edges of the fixture to create a special atmospheric effect

- Non-dimmable, 3-in-1 dimmable (1-10V/PWM/Resistor) and DC input options

- Slim, compact and fashionable design

- Bundled LED driver with Active PFC and OTP/SCP

- Very low power consumption

- No UV or IR radiation

### Read more about this new product in GlacialLight's website:

http://www.glaciallight.com/products/downlight-recessed.htm

### About GlacialLight

GlacialLight, the lighting division of GlacialTech, manufactures LED lighting solutions for indoor/outdoor applications and both residential and commercial uses. As well as having a wide range of finished LED lighting products, GlacialLight also offers its clients the option of customizing products for specific needs. Please visit the company homepage at www.GlacialLight.com