# GlacialPower Announces a Series of Compact LED Drivers

## Four New Tiny LED Drivers with High Power Efficiency

9 August, 2012, Taipei, Taiwan - GlacialPower a sub-division of the experienced technology manufacturer GlacialTech Inc., is pleased to announce a compact GlacialPower LED drivers with high power efficiency – GP-LS50PH-142C, GP-LS70PH-100C, GP-LS100PH-142C and GP-LS100PH-71C.

With the compact dimensions of 177 x 57 x 37mm (LxWxH), these 4 LED drivers make LED lighting installations even easier compared to bulkier drivers. Even while being small, this series has a high power conversion efficiency rate of 91% ensuring low power consumption.

These four fully integrated AC/DC LED driver micro-modules incorporate the active PFC circuitry to deliver up to 99.4W of power to the LED with a high power factor. With the current trend for higher output voltage requirements, due to more and more applications using LEDs in a series, this series provides output voltage up to 142V DC.

GlacialPower's R&D team has developed the LED driver to eliminate the need for an electronic transformer, aiming to provide more energy-saving and environmentally-friendly LED products. GlacialPower's LED driver technology is expected to lead a new trend in innovation and become an essential feature for all LED lighting products.

### Features

- Universal AC input 90 ~ 305V AC

- High Efficiency up to 91%

- High Output voltage

- Constant current operation

- Built in Active PFC function

- Protection: OVP/ STP/ OTP

- IP67 Design

- Suitable for LED stripe

- Cooling by fresh Air confection

- Compact Size

### Specifications

| Model No. |  | GP-LS50PH-142C | GP-LS70PH-100C | GP-LS100PH-142C | GP-LS100PH-71C |
| --- | --- | --- | --- | --- | --- |
| Output | Rated Voltage | 142V | 100V | 142V | 71V |
|  | Constant Current Region | 100~142V | 50~100V | 100~142V | 50~71V |
|  | Rated Current | 350mA | 700mA | 700mA | 1400mA |
|  | Current Tolerance | +5%,-5% |  |  |  |
|  | Rated Power | 49.7W | 70W | 99.4W | 99.4W |
|  | Ripple&Noise(Max.) | 700mV | 700mV | 1000mV | 500mV |
|  | Efficiency at 230Vac(Typ.) | 89% | 90% | 91% | 91% |
|  | Set up Voltage | -1%~5% |  |  |  |
|  | Set up time (Max.) | 2000ms / 240 V AC at full load. |  |  |  |
| Input | Rated Voltage | 100 ~ 277 V AC |  |  |  |
|  | Voltage Range | 90 ~ 305V AC / 127 ~ 431V DC |  |  |  |
|  | Frequency Range | 47 Hz ~ 63 Hz |  |  |  |
|  | AC Current (Max.) | 1.2A at 120V AC / 0.6A at 240V AC |  |  |  |
|  | Power Factor | ≧0.98 at 120 V AC / ≧0.94 at 240V AC at full load, PF≧0.9 at 70% load |  |  |  |
|  | Inrush Current (Max.) | COLD START 50A at 240 V AC |  |  |  |
|  | Leakage Current | <0.75mA / 230 V AC |  |  |  |

### Excellent LED lighting products you can trust

Design of LED lighting products is based on three core technologiesincluding electrical design (LED drivers), mechanical design (cooling devices), and optical design (lamp holders), respectively performed by GlacialPower, GlacialTech, and GlacialLight in the GlacialTech family. With the three core technologies in hand, GlacialTech has integrated all the resources required for exceptional designs and manufactures excellent LED lighting products you can trust.

### About GlacialLight

GlacialTech Inc., is a diversified provider of cooling, power supply, and PC enclosure solutions for consumer and industrial applications. Having established strong relationships with LED semiconductor Co.’s, thermal, SMPS and mechanism technology leaders worldwide, GlacialTech leverages world-class engineering, efficient manufacturing and highest-quality materials to provide high brightness, low power LED lighting solutions for indoor, outdoor and other customized applications under the GlacialLight brand. Please visit the company homepage at www.GlacialLight.com.