# GlacialPower Launches GP-LS45P Series: 40-48 Watt LED Drivers With IP67 Rating

## With Active PFC, 90 to 305 VAC Input, 24 to 57 V DC output, and Three Dimming Modes

August 25, 2016 - Taipei Taiwan - GlacialPower, a division of LED technology manufacturer GlacialTech Inc, is pleased to announce a new series of 40 to 48 Watt LED Drivers with DC output ranging from 24 V DC up to 57 V DC. The GP-LS45P series accepts a wide input range from 90 to 305 VAC, to handle AC supply fluctuations. 

Suitable for interior and exterior use, the GP-LS45P drivers are IP67 rated for dustproofing and waterproofing. Active power factor correction makes efficient use of mains electricity. The driver supports a constant current mode supporting three optional types of dimming. There are five models in the GP-LS45P series, which provide DC output of 24 V, 36 V, 42 V, 48 V and 57 V respectively, and output power ranging from 40 up to 48 Watts.

### Efficient LED Drivers with Dimming Options

The GP-LS45P drivers not only provide active power factor correction for efficient use of mains power, they also provide high power output efficiency. These drivers are fully compliant with Stage 3 of the stringent ErP (EU) No. 1194/2012 directive, which aims to greatly enhance the efficiency of energy-related products.

For easy integration and maximum efficiency there are three dimming mode options available: DC or electronic potentiometer dimming from 0-10 V, PWM dimming from 0-10 V, and 0K to 100KΩ resistance dimming. As well as running very efficiently in normal operation, the GP-LS45P series drivers also use almost no electricity in standby mode – they provide a no load consumption of less than 0.5W at 230 VAC.

### Smooth Output Voltage for Long LED Lifetime

Output voltage from the GP-LS45P series is smooth and stable, as shown by the minimal noise and ripple specifications: from 200 mV to no higher than 250 mV, depending on the model, resulting in steady and consistent LED lighting. These drivers also help your LEDs start up in an instant, with a setup time of less than 0.5 seconds at 230 VAC.

### Tough and Reliable LED drivers

The GP-LS45P series of drivers have a very wide operating temperature range, from -30° C to 50° C. In addition, these LED drivers' rugged IP67 rated enclosures are waterproof and dustproof enclosure to ensure integrity and stability in challenging indoor and outdoor environments. Safety features include Over-Voltage Protection (OVP), Over-Current Protection (OCP), Short-Circuit Protection (SCP) and Over-Temperature Protection (OTP), ensuring both the safety and long service life of this LED driver.

### Features:

- Universal AC input 90~305VAC.

- Built-in Active PFC Function.

- Protections: OVP, OCP, SCP and OTP.

- 3 in 1 dimming option (DC 0-10 V / PWM / Resistor) in constant current mode.

- IP67 rated enclosure.

- Compliant with ErP EU_1194_2012, Stage 3 directives for energy related products.

- No load consumption is less than 0.5 W.

- Set up time is less than 0.5 Sec at 230VAC.

### Specifications

| Model No. |  | LS45P-24C(A) | LS45P-36C(A) | LS45P-42C(A) | LS45P-48C(A) | LS45P-57C(A) |
| --- | --- | --- | --- | --- | --- | --- |
| Output | DC Voltage | 24V | 36V | 42V | 48V | 57V |
|  | Constant Current Region | 16 ~ 24V | 24 ~ 36V | 33 ~ 42V | 33 ~ 48V | 40 ~ 57V |
|  | Rated Current | 1.85A | 1.25A | 1.05A | 1A | 0.7A |
|  | Current Tolerance | -5% ~ +5% |  |  |  |  |
|  | Rated Power | 44.4W | 45W | 44.1W | 48W | 40W |
|  | Ripple & Noise (Max.) | 200mV | 250mV | 250mV | 250mV | 250mV |
|  | Efficiency at 240Vac(Typ.) | 85% | 86% | 86% | 87% | 87% |
|  | Voltage Tolerance | -1% ~ +5% |  |  |  |  |
|  | DC Dimming (Optional) | DC 0-10V or Electronic Potentiometer 0-10V |  |  |  |  |
|  | PWM Dimming (Optional) | Puls: Hi=10V Low=0V, Duty: 0%-100%, Fsw:0.5-3KHZ |  |  |  |  |
|  | Resistance Dimming (Optional) | 0K-100KΩ |  |  |  |  |
|  | Set up time (Max.) | < 0.5 Sec / 230VAC at full load |  |  |  |  |
|  | No Load Consumption (Max.) | < 0.5W at 230 VAC |  |  |  |  |
| Input | Voltage Range | 90VAC ~ 305VAC |  |  |  |  |
|  | Rated Volatge | 100 ~ 240VAC / 277VAC (for North American Only) |  |  |  |  |
|  | Frequency Range | 47 Hz ~ 63 Hz |  |  |  |  |
|  | AC Current (Max.) | 0.6A at 100 ~ 240VAC / 0.25A at 277VAC for North American Only |  |  |  |  |
|  | Power Factor | ≧ 0.98 at 120VAC / ≧ 0.93 at 240VAC/ ≧ 0.9 at 277VAC, at full load |  |  |  |  |
|  | Inrush Current (Max.) | COLD START 70A at 240VAC |  |  |  |  |
|  | Leakage Current | < 0.5mA / 240VAC |  |  |  |  |

### Read more about GP-LS45P in GlacialPower's website:

http://www.glacialpower.com/products/led-driver-PFC2.htm#L6

### About GlacialPower

GlacialPower, the power division of GlacialTech, manufactures Switch Mode Power Supply (SMPS) solutions for LED lighting applications. GlacialPower provides LED Drivers and power supplies with a wide wattage range for a variety of applications. GlacialPower power supplies use attractive and contemporary design to deliver quality products that possess innovative features and ease of usability for customers. For details, please visit at www.GlacialPower.com or visit us on Facebook at www.GlacialPower.com or visit us on Facebook at www.facebook.com/GlacialTech/.