FOR IMMEDIATE RELEASE

# Lian Li Releases the PC-10N Mid Tower Chassis in North America

## Side Mounted Radiator Support and Railing Motherboard Tray Design

24 September 2013, Keelung, Taiwan - Lian-Li Industrial Co. Ltd today releases a new brushed aluminum mid tower chassis with side mounted radiator support, PC-10N, in North America. Besides allowing for a 240mm radiator to be placed on a hinged bracket, the PC-10N forgoes the traditional motherboard tray, and instead implements a railing mount design for increased thermal performance and better cable management.

### Side Mounted 240mm Radiator Support

A hinged bracket directly across the CPU allows DIY builders to install a 240mm radiator in the PC-10N. The bracket secured by thumbscrews can be swung open on its hinge to allow for easy access of the internal components.

### Railing Mount Motherboard Tray

Lian Li's railing mount technology greatly improves airflow under the motherboard, helping systems run cooler. This design replaces the traditional solid motherboard tray which can block airflow and adds to system weight and cost. The railing system also provides better routing options for internal wiring and cables, reducing clutter, improving system appearance and further improving airflow.

### Tool-Less Hard Drive Installation

Hard drives can be tool-lessly installed in the PC-10N. The six 3.5” drives and one 2.5” drive are secured in the two HDD cages by quick-access thumbscrews with rubber suspensions to greatly reduce irritating noise and vibrations. An additional 2.5” can be installed using four standard screws. Additionally, the top HDD cage can be removed tool-lessly allowing for increased airflow and more space for longer expansion cards.

### Thermal Design

Two 120mm fans with air filters located in the front of the chassis pull in cool air. Once the front bezel is tool-lessly popped off, the front fans can be effortlessly removed. The rear 120mm exhaust fan pulls hot air out of the chassis, and an additional 120mm fan can be installed in the top panel.

### Compatibility

VGA cards up to 300mm (11.81”) in length can fit in the PC-10N when the top HDD cage is not removed, and 430mm (16.9”) when the cage is. CPU coolers up to 155mm are supported.

The Lian Li PC-10N also supports power supplies up to 300mm (11.81 inches). Ventilation slits underneath the PSU, with a removable pull-out air filter, help increase the life and performance of the power supply and system.

### Connectivity

The front I/O panel is stylishly placed on an angle on the right side of the front panel and contains two USB 3.0 ports and HD audio connections.

The PC-10N is available in two color options: silver (PC-10NA) and black (PC-10NB).

### Price and Availability

The PC-10N will be available on Newegg for the suggested retail price of $129.99.

More product information and pictures can be found here:
http://www.lian-li.com/en/dt_portfolio/pc-10N/

Check out Lian Li's Facebook page here:
http://www.facebook.com/LianLiHQ

Video product tour of the PC-10N:
http://youtu.be/xrcRn4ityGI

### About Lian Li

Lian Li Industrial Co. Ltd is an Aluminum Chassis and Chassis peripheral manufacturer and provider of industrial wholesale as well as OEM & ODM services for the PC industry as required. With over 20 years of service excellence, ISO 9001 certification and a team of professional staff willing to do the utmost to deliver durable, superior quality products, all backed by a *2 year guarantee; your satisfaction is guaranteed (*conditions apply). To learn more about Lian Li, please visit their websites at www.Lian-Li.com