FOR IMMEDIATE RELEASE

# Lian Li Announces PC-7H Mid Tower Chassis

## Elegant Design, Simple Functionality

October 2, 2012, Keelung, Taiwan - Lian-Li Industrial Co., Ltd, today announces a new brushed aluminum mid tower chassis – PC-7H. Supporting ATX and micro-ATX motherboards, the elegant PC-7H gives DIY builders a simple yet functional design with many tool-less options. 

The Lian Li PC-7H can hold up to four 3.5” hard drives and one 2.5” SSD in the HDD cage in the front of the chassis. The 3.5” drives are secured into place using thumbscrews with rubber suspensions to reduce vibrations. For those looking to modify this all aluminum chassis, the HDD cage can be easily removed. 

Above the HDD cage are the three 5.25” drive bays. Thanks to the tool-less design of the bays, DIY builders can easily install optical drives.

Two 140mm fans in the front of the chassis pull cool air into the chassis. The bottom 140mm fan blows air directly on the HDD cage, while the upper fan pushes air across the motherboard and expansion cards. The front panel is tool-lessly removable, making it easy to clean and maintain the air filters. The rear 120mm exhaust fan pulls hot air out of the chassis.

For those looking for more cooling options, there is space on the top panel for two 120mm fans, and more room for another 120mm fan on the side panel.

The motherboard tray features several cutouts for easy cable management, and the PC-7H accommodates CPU coolers up to 170mm (6.6”) in height. The PC-7H has 8 vented expansion slots with thumb screws. Expansion cards up to 400mm (15.7”) are supported.

Power supplies up to 250mm (9.8”) in length easily fit at the bottom of the PC-7H. Ventilation slits underneath the PSU with a removable pull-out filter help increase the life and performance of the power supply.

The I/O panel sits on the top of the chassis and features two USB 3.0 and HD audio connections. A door covers the I/O panel to keep the elegant look when not in use. 

The PC-7H comes in three color options: silver (PC-7HA), black (PC-7HB), and interior black (PC-7HX).

### Specifications:

| Model | PC-7H |
| --- | --- |
| Case Type | Mid tower chassis |
| Dimensions | (W) 210mm x (H) 473mm x (D) 490mm |
| Front bezel Material | Aluminum |
| Color | Internal Black / Silver / Black |
| Side Panel | Aluminum |
| Body Material | Aluminum |
| Net Weight | 5.8KG |
| 5.25" drive bay (External) | 3 |
| 3.5" drive bay (External) | None |
| HDD bay | 3.5-Inch HDD x 4 / 2.5-Inch HDD x 1 |
| Expansion Slot | 8 |
| Motherboard | ATX / Micro ATX |
| System Fan (Front) | 140mm Fan x 2 |
| System Fan (Top) | None |
| System Fan (Rear) | 120mm Fan x 1 |
| I/O Ports | USB 3.0 x 2 / HD Audio |
| Maximum Compatibility | VGA Card length: 400mm PSU length: 250mm CPU cooler height: 170mm |

More product information and pictures can be found here:
http://www.lian-li.com/v2/en/product/product06.php?pr_index=631&cl_index=1&sc_index=25&ss_index=62&g=f 

Check out Lian Li's new facebook page here:
http://www.facebook.com/LianLiHQ
  

PC-CK101 Facebook Giveaway
Along with the launch of the new facebook page, Lian Li will be giving away a PC-CK101 to one lucky fan. To enter you only have to go to the “PC-CK101 Contest” tab on Lian Li's facebook page, www.facebook.com/LianLiHQ, and press like. More rules and details are available there.

### About Lian Li

Lian Li Industrial Co. Ltd is an Aluminum Chassis and Chassis peripheral manufacturer and provider of industrial wholesale as well as OEM & ODM services for the PC industry as required. With over 20 years of service excellence, ISO 9001 certification and a team of professional staff willing to do the utmost to deliver durable, superior quality products, all backed by a *2 year guarantee; your satisfaction is guaranteed (*conditions apply). To learn more about Lian Li, please visit their websites at www.Lian-Li.com